KLH

DEPARTMENT OF ELECTRONICS & COMMUNICATION ENGINEERING

RESEARCH PAPER REVIEW AND LITERATURE SURVEY

Temporal Fusion Transformer-Based Multi-KPI Forecasting for Proactive Open RAN Control

Submitted by
B. Chandra Nikhilesh | 2520040091
M. Sujeeth Reddy | 2520040025
N. Sai Charan | 2520040021

Project Submission | Academic Year 2026–2027

# Abstract

This focused literature survey reviews work directly relevant to multi-KPI forecasting for Open RAN using a Temporal Fusion Transformer (TFT). It covers TFT architecture, recent forecasting-to-control research in O-RAN, public RAN KPI datasets, and O-RAN controller applications. The principal gap is demonstrating reliable multi-horizon predictions with calibrated uncertainty and useful early-warning lead time on network telemetry. The current student prototype uses synthetic data; it is not live-network validation.

# 1. Review scope and method

Sources were selected for direct relevance to the project’s five KPIs (throughput, latency, packet loss, PRB utilisation and active UEs), 12-hour input window, 3-hour forecast horizon, P10/P50/P90 outputs, and future proactive-control direction. This is a focused narrative review, not an exhaustive systematic review.

# 2. Research papers and datasets

## 2.1 Lim et al. (2021), “Temporal Fusion Transformers for Interpretable Multi-horizon Time Series Forecasting,” International Journal of Forecasting 37(4), 1748–1764. DOI 10.1016/j.ijforecast.2021.03.012.

Relevance: TFT combines variable selection, gated residual blocks, recurrent processing for local patterns, interpretable attention for longer-range relationships and quantile forecasts. This directly supports the project’s multi-step P10/P50/P90 design.

Limitation for this project: It is a general forecasting paper, not a telecom validation study. Performance claims from its other datasets cannot be transferred to O-RAN KPIs.

## 2.2 “Forecasting before acting: Recent advances in learning-based traffic prediction and intelligent control in Open RAN” (ICT Express, 2026), DOI 10.1016/j.icte.2026.07.007.

Relevance: This O-RAN-focused review studies how traffic and UE predictions feed downstream control. It highlights differences in prediction granularity, ML architecture, evaluation environment and O-RAN interface placement.

Limitation for this project: The review reports that evaluation often relies on custom simulators and that live closed-loop actuation evidence is limited. Therefore this project must distinguish forecast alerts from actual control.

## 2.3 VERGE/CTTC, “OAI RAN KPM dataset,” public dataset; Zenodo DOI 10.5281/zenodo.17935683.

Relevance: Time-series metrics collected using OpenAirInterface, FlexRIC and a KPM xApp in a 5G emulation environment over about five weeks. Fields include latency, PRB allocation, PDCP volumes, RLC delay and UE throughput.

Limitation for this project: This is experimental 5G testbed/emulation data, not a commercial live operator feed. Its schema does not automatically contain every project KPI; inspect units and map fields without inventing missing measurements.

## 2.4 O-RAN Software Community, “Simulated datasets” (updated 2024).

Relevance: Documents historical Telecom Italia internet-traffic data at 10-minute intervals and a separate Viavi synthetic dataset with 4G and 5G cell/UE reports.

Limitation for this project: The Telecom Italia data represents historical aggregate traffic, not all cell-level PRB/latency/loss/UE KPIs. The Viavi dataset is synthetic and may require access approval/password.

## 2.5 O-RAN Software Community, RIC application documentation: KPI Monitoring, QoE Prediction and Traffic Steering xApps.

Relevance: Shows a reference pattern in which RAN KPI monitoring supplies telemetry and prediction outputs can inform traffic-steering decisions.

Limitation for this project: Documentation does not mean this student prototype already implements an xApp or controls a network.

# 3. Method comparison

| Method/data | Contribution | Relevance | Caution |
| --- | --- | --- | --- |
| TFT | Multi-horizon probabilistic forecasts; interpretable components | Core model for five KPIs | Must be validated on network telemetry |
| Persistence | Repeats last observed value | Simple baseline | May fail during load bursts |
| LSTM | Sequential deep-learning baseline | Model comparison | Not inherently calibrated quantile output |
| OAI/FlexRIC KPM | Experimental 5G RAN measurements | Closest candidate for radio KPI validation | Field/unit mapping required |
| Telecom Italia traffic | Real historical aggregate traffic | Demand-pattern research | Not a complete radio KPI dataset |
| Current project CSV | 1,600 generated rows reported by terminal | Tests app and pipeline | Synthetic, not live data |

# 4. Research gap

Jointly forecasting multiple related KPIs may capture interactions missed by isolated single-KPI models.

Low average error alone is insufficient; a proactive system also needs uncertainty calibration and event-level warning evaluation.

Threshold crossing is a risk indicator, not proof that a cell is overloaded; thresholds require evidence from labelled congestion periods.

Forecasting and network control are separate stages. Safe xApp control needs a permitted testbed, guardrails, operator approval and rollback.

Simulation results may not generalise to real 4G/5G environments with different load, radio conditions and equipment.

# 5. How the project addresses the gap

The prototype uses 48 historical 15-minute steps (12 hours) to forecast 12 steps (3 hours) for throughput, latency, packet loss, PRB utilisation and active UEs. P10/P50/P90 quantiles provide a central estimate and uncertainty band. The dashboard uses upper-quantile thresholds for early-warning messages and compares TFT with persistence and LSTM on simulated data. The next research step is to test the same pipeline on a documented OAI/FlexRIC dataset and evaluate warning precision, recall, false-alarm rate and lead time.

# 6. Recommended experiments

Audit dataset licence, timestamp cadence, units and column meanings.

Keep chronological train/validation/test splits; fit scalers on training data only.

Report MAE and RMSE per KPI and per forecast horizon; use MAPE carefully near zero.

Evaluate P10–P90 coverage together with interval width.

Create congestion labels from documented events and measure warning precision, recall, false alarms and lead time.

Compare TFT, persistence and LSTM on identical timestamps/windows.

Keep the app advisory-only until testbed validation and control safeguards are complete.

# References

- References

- Lim, B., Arık, S. Ö., Loeff, N., & Pfister, T. (2021). Temporal Fusion Transformers for interpretable multi-horizon time series forecasting. International Journal of Forecasting, 37(4), 1748–1764. https://doi.org/10.1016/j.ijforecast.2021.03.012

- Dang, T. T.-A., et al. (2026). Forecasting before acting: Recent advances in learning-based traffic prediction and intelligent control in Open RAN. ICT Express, 12(4), 1041–1061. https://doi.org/10.1016/j.icte.2026.07.007

- VERGE Project. (n.d.). OAI RAN KPM dataset. GitHub. https://github.com/VERGE-PROJECT/OAI_RAN_KPM_dataset

- O-RAN Software Community. (n.d.). Traffic Steering xApp: User’s Guide. https://docs.o-ran-sc.org/projects/o-ran-sc-ric-app-ts/en/latest/user-guide.html

- Google Research. (n.d.). Temporal Fusion Transformers for interpretable multi-horizon time series forecasting. https://research.google/pubs/temporal-fusion-transformers-for-interpretable-multi-horizon-time-series-forecasting/